136kHz portable transmitter.

The transmitter described here has been developed over the last few months of operating on 136kHz and incorporates a number of refinements to the original, very simple design.

In order to radiate a useful signal on 136kHz one either needs a superb aerial and a few Watts or a normal sized aerial and a few hundred Watts. Portable operation with long wires or kite-supported aerials can be a way for the operator with a small garden to get on the band. A 100mtr end fed wire, at a sensible height, will have a radiation efficiency of a few points of a percent requiring 500W or so to give the allowed 1W ERP. These factors give the design brief for the TX, it must produce as near to 500W as possible, it must run off a car battery, it must be small and it must be capable of withstanding misuse.

A quick calculation tells us that a 12V operated TX producing 500W must draw 50A or more from its supply so we must use some pretty hefty output devices. I have been using power mosfets in 160/80 linear amps for a few years and decided to try them in this application which is much closer to their intended role.

Circuit description.

The rig is crystal or VFO controlled and uses Philips Locmos 4000 series chips to generate push-pull square-waves on 136kHz from the high frequency source. The basic divider chain requires an input at 1.36Mhz which comes either directly from the VFO or from the 13.6Mhz crystal via a further divider. The 4013 D-type must divide by two in order to produce its push-pull output so the 4017 decade counter is strapped to divide by five by connecting Q5 back to the reset pin. The 136kHz square-waves are fed to the TC4426 driver chip via 1k resistors so that undue loading of the 4013 outputs does not take place when the driver chip is has no supply (key up). An interlock to prevent damage to the output devices, should the drive fail, is provided by the 4538 re-triggerable monostable. This chip is clocked by the 272kHz signal at the output of the 4017 and, if the signal stops, the output goes low turning Q4 on and preventing keying via Q3.

For stability, the VFO or crystal oscillator is left running all the time that the rig is on. It causes no QRM because no 136kHz signal is generated until the divider chain is energised when switching to TX mode.

When the key is pressed and drive is present, Q3 is turned on allowing 12V to be applied to the driver chip, which drives the output FETs into conduction. Some may wonder why I use a complex keying system rather than just passing the drive through a gate. The answer is key-clicks. If the TX were to be turned on and off at the rise-time of the drive waveform the sidebands, even after the filtering provided by the high Q aerial, would extend well outside the band. Q3 turns on and off fairly slowly and the gentle rise and fall of RF is aided by the fact that the output mark space ratio of the 4426 gets less as the volts drop…. Don't use its' non-inverting sister, the 4427 as the mark space ratio increases on that one and the drive waveforms overlap.

The power mosfets are operating in class D/E, either hard on or turned off and so dissipate very little power and only require a small heatsink. They must be kept reasonably cool, however, as their internal resistance rises with temperature.

The output transformer steps the voltage up by the amount chosen by the output selector switch. The more turns on the secondary, the more voltage and hence the more power into 50 Ohms.

Up until now we have been dealing with square-waves which are rich in odd harmonics so an output filter is required before we dare apply this to an aerial. This is the part of the design that has given the most problems, there's a lot of harmonic energy to dissipate and some heating of components will take place. The filter roll-off frequency is set quite high at about 220kHz as there is virtually no second harmonic to worry about. When used in conjunction with the usual high Q aerial tuning system the harmonic output is well supressed. Those of you with full-sized quarter wave aerials and no ATU may require more filtering!

The receive pre-amp uses a pair of Toko 12VX coils in a top-coupled band pass filter with a j-fet to make up for the insertion loss. If more gain is required, make the j-fet an amplifier with 100R in its source and 1k in its drain and connect the drain straight to the base of a BC183 emitter follower with a 470R emitter resistor.

Parts list.

Component		Qty.	Supplier	Part No.	Approx price

HEF4001		1	Farnell		384-434	60p

HEF4017		1	Farnell		384-720	£1

HEF4013		1	Farnell		384-630	70p

HEF4538		1	Farnell		386-339	£1.50

TC4426		1	Farnell		295-024	£1.80

Q3, ZTX718		1	Farnell		663-244	50p

Q4, BC213B		1	Farnell		356-700	8p

Q1/2, HUF75343G3	2	Farnell		247-042	£3       see note *

Q5, BF245A		1	Farnell		352-044	35p

CH1 core		1	Farnell		559-570	£2

T1 core		1	Cirkit		55-03801	£10	see note **

TX LPF cores 		2	Mainline	T200-2		£4

RX filter coils (Toko)	2	Bonex		719VXA-A017AO	72p

RX filter 56pF C	1	Farnell		105-886		15p

RX filter 2200pF C	2	Farnell		105-889		22p

TX filter 10nF C	2	Farnell		106-370		30p

TX filter 4n7   C	1	Farnell		106-363		30p

TX filter 2n2   C	2	Farnell		106-362		25p

TX filter 22nF C	1	Farnell		106-371		40p

Decoupling 2u2 C	1	Farnell		286-928		£1.50

0.1uF decouplers	10	Farnell		143-680		27p

Relay			1	Farnell		910-739		£2.30

Case			1	Farnell		518-876		£13

VFO tr. BC183	1	Farnell		356-580		15p

1mH choke		1	Farnell		608-609		25p

22pF  C		1	Farnell		105-056		20p

330pf C		1	Farnell		105-063		20p

1nF  C			6	Farnell		105-066		20p

*Note re FETs. Slightly better devices with a lower “RDS on” are available; type HUF75345G3, part no. 247-030 at £5.42 each plus VAT.

**Note re T1. If you can source Philips ferrites, the 58mm toroid type 58/40/17.5 3C85 is a much better core than the Cirkit one and will stand 1kW output! (Cirkit now seem to have a £50 min order so that rules them out anyway…) They are available from Arrow on 01279 626 777 part no. 420605G at £6.23 plus VAT each. Unfortunately they have a minimum order of £25 but if enough people want them I will buy a quantity and supply them at cost.

Contact me by e-mail at G3YXM@picks.f9.co.uk or by post at the address at the end of the article.

Unspecified parts, resistors and capacitors etc. are standard types.

You will also need;

Some 0.4W resistors.

VFO variable capacitor, about 100pF

Heat sink.

Insulating washers for fets.

Connecting wire and thick power wire.

35A or 50A car fuse and holder

Diode for relay.

TX switch.

Meter and shunt for 50A full scale.

Key, aerial and rx sockets.

Silcone rubber sealer.

Double sided copper-clad board.

5 way rotary switch.

Short length of 5-core braid-screened cable.

Suppliers.

Farnell 	01132 636 311

Cirkit		01992 444 111

Bonex		01753 549 502

Keytronics	01279 505 543

Mainline	01162 777 648

Construction.

The VFO is constructed in a small box made from pieces of double-sided copper-clad board soldered together. Leads of components should be kept short and a bit of silicone rubber or candle wax could be applied to hold parts firmly to prevent frequency wobble. The VFO frequency will be divided by ten so a hundred Hertz drift at VFO frequency will only be 10Hz at output frequency. Even so it is worth using good quality polystyrene Cs in the VFO to counter the positive temperature coefficient of the other components. The VFO should be routed to the 4001 squarer via a short piece of coax. This gate, strapped as an amplifier, has a very high gain, leads must be kept short and  decoupling Cs applied across each chip to guard against instability.

I built the CMOS circuitry on strip-board taking care to keep the tracks short and earth all unused inputs.

The TC4426 chip is capable of driving 1.5A into the gate capacitances of the big FETs and the decoupling Cs must be fitted close to the chip with short leads. The 6R8 series resistors are actually mounted on the gate pins of the FETs, the resistor leads forming the connections to the strip board. The strip board should be grounded to the earth plane as near to the FETs as convenient via a short lead.

Be careful to ensure that the power supply to the low power stages is very well decoupled from the high power section. If you are using the rig at home you can get away with a simple, unregulated but well smoothed, 12V high-current supply for the output stage and a small regulated (250mA or so) PSU for the other bits.

The FETs are static sensitive devices and can be destroyed by careless handling.

They should be mounted so that the source leads are soldered straight down onto the ground plane. It is good idea to solder the 47k resistors from each gate to ground first, to prevent static build-up during construction.

Having applied a very small amount of heat-sink compound to both sides of the transistor insulating washers, the FETs should be clamped firmly down to the heat sink and a test made to check that the drains are not shorted to it.

The output transformer (T1) should be constructed from a few inches of five-core screened cable wound four times through the ferrite toroid. Use a cable with a thick braided screen as this forms the four-turn primary and carries a lot of current. The inner cores are connected in series to form the secondary. A switch selects the appropriate tap (joints between inner cores) to produce the amount of power desired. The primary centre-tap is made by carefully removing some of the outer plastic sleeve to expose the braid and soldering a thick wire onto it. Take care to tease the braid away from the internal conductors before soldering or you may melt the insulation.

The wiring between the transformer and the switch will carry high peak voltages and should be well insulated and screened from the low-power circuitry. Similarly the output filter, which should be constructed on a separate piece of copper-clad board.

The most critical components are the dust-iron cores, T200-2s may be expensive and require a lot of turns but they work better than most. To get the correct inductance you will need about 65 turns of 1mm wire. Use a small computer fan  running on TX +12V to keep the toroids cool. I mounted the toroids on foam pads and stuck them down with silicone rubber. It is difficult to get suitable Cs in the values required so the filter Cs are made up from smaller ones, the 12nFs are 10+2n2 and the 27nF is 22+4n7.  At the filter output there should be a good sine-wave at up to 350W into 50 Ohms. Large deviations from 50 Ohms will cause the filter components to get hot.

Remember; the source and drain connections to the output devices have to carry up to 70A and must be made using thick wire and good solid joints! Very thick power cables should be used to connect the rig to a battery, power drops off quickly if the Volts drop much below 12. The HUF75345G3 and similar devices must not be used above about 15V supply.

Testing.

Get the VFO and CMOS stages working first, you should be able to hear the 136kHz signal from the 4013 on your RX by placing a probe near the chip. You can safely key the drive into the mosfets with no power applied to them (remove the fuse). Check with a scope that you have complimentary 12V square waves on the two gates, don't worry if the waveform is a little rounded off, it is due to the high gate capacitance of the FETs.

Connect the TX to a 50 Ohm load and, having selected the first tap on SW2 (one-turn secondary) apply power to the FETs via a 12V car bulb or something similar to limit the current. The FETs should draw no current when there is no drive. Press the key and the output stage should draw a few amps and produce about 15W into the dummy load. Any sparks or smoke at this stage is a bad sign……

If all seems well you can remove the current limiting and continue tests, increasing the power by selecting taps, key the rig in short bursts and check for overheating of FETs and cores, there should be none yet.

Never key the rig into open circuit, especially at the high power settings. Do not turn the tap switch under power, it will spark and may damage the contacts and possibly the FETs!

The TX is sensitive to load and some method of measuring the impedance of the aerial system is desirable. Most good SWR bridges will give a useful indication of match at 136kHz but the power readings will be low. My Diawa reads 20W forward power when the rig is generating 350W! Always monitor the current to the output stage, try to keep the key-down current below 60A to be sure of staying within the limits of the FETs.

The receive band-pass filter will need tuning up. Try and use a signal near the centre of the band when adjusting as it is quite sharp.

Expected output power at the different switch settings with a 12V supply (approximate):

Tap 1 ( one turn secondary ) = 12W

Tap 2 ( two turn secondary ) = 50W

Tap 3 ( thre turn secondary ) = 110W

Tap 4 ( four turn secondary ) = 200W

Tap 5 ( five turn secondary ) = 310W

Possible variations

Any reasonable power FETs should work, a low rds-on (less than 0.01 Ohm) is desirable, a suitably large current rating and a Voltage rating of four times the supply rail. The HUF75343G3 FETs from Farnell are only 55V rated so cannot be used above about 14V. They do have the advantage of a very low rds-on which makes them efficient. You could use any similar 75A device such as the Ixys IXFH75N10 which is available from Keytronics at £6 each. These are 100V devices and could stand a supply of up to 25V.

Connecting four FETs, two in parallel on each side, will give more power and make the rig nearer to indestructable!

Any even multiple of output frequency could be used as a drive source, the 4013 must be left to do its divide by two but any other division ratio could be used ahead of it. It may also be OK to take in 136kHz and generate the complimentary square waves with a squarer and inverter….

A 73 kHz version works fine using exactly the same circuit, just double all the component values in the output filter.

Warning: If you apply 300W or so of 136kHz to a high Q aerial system, the Voltage on the wire will be over 10kV. Most insulated wire will break down and arc across to anything nearby. Keep all aerial wires well clear of everything and use good insulators. RF burns hurt, I know!

Good luck.

If you have any problems or suggestions e-mail me at  G3YXM@picks.f9.co.uk

© G3YXM 1999

Updated 1/1/2000

Dave Pick

178 Alcester Road South

Kings Heath

Birmingham

B14 6DE